| Name | Security Level |
| --- | --- |
| BELLSING® BSP dynamic speaker | Open |
| Version | A Total of 9 Pages |
| REV A |  |

D11 dynamic speaker

BSP110UAB16

Product Data Sheet

| Prepared by: | Jerry |  | Date: | 2021-09-18 |
| --- | --- | --- | --- | --- |
| Auditor: | Allen |  | Date: | 2021-09-19 |
| Approver: | Allen |  | Date: | 2021-09-19 |

Shenzhen Bellsing Acoustic Tech. Co., Ltd.

Restricted

## 1.Security Warning

The information contained in this document is the exclusive proprietary to Bellsing Inc., and should not be disclosed to any third party without the written consent of Bellsing Inc.

## 2.Environmental Requirement

This s product including all components must be free from lead (Pb) and other banned substances according to customer's requirements.

## 3. Record Update

| Date | Version | Description | Editor |
| --- | --- | --- | --- |
| 9/18/2021 | A | Initial Release | Jerry |

BSP110UAB16 dynamic speaker

# Description

## This product works on the principle of permanent magnet. When electro-magnetic field and voice coil, on which a vibration membrane is attached, receive a current signal, the membrane vibrates and the air around the diaphragm will be compressed. Sound will be induced through such kind of compression and then the electronic sig- nal is transformed into sound signal.

## The BSP110 is a high performance Φ11mm dynamic speaker, with stronger bass by ultra Xmax technology. Recommaned to be used in consumer electronics inear products, especially for Active Noise Cancellation application.

# Mechanical Specification

## Mechanical dimension：please refer to the drawing below

# Electro-acoustic Specifications

## 1.3.1 Operation and storage environment conditions

## Operating Temperature Range: -20℃ to +70℃

## Storage Temperature Range: -40℃ to +85℃

## 1.3.2 Test Method

## 1.3.3 Electric-Acoustic Specification

| Test Item | Nominal | Tolerance | Comments |
| --- | --- | --- | --- |
| Impedance | 16 | ±10% |  |
| DC resistance | 15 | ±10% |  |
| Rated power | 5mW |  |  |
| Maximum shor-term power | 10mW |  |  |
| Resonance Frequency (F0) | 390±30Hz in free air |  | 126mVrms |
| Sound pressure level | 110±3dB at 1kHz |  | 126mVrms IEC711 |
| SPL | See Figure1 |  | 126mVrms |
| THD | See Figure2 |  | 126mVrms |
| Frequency Range | 20~20kHz |  |  |
| Rub & Buzz | Must be free audible noise from 50 Hz to 3KHz |  | 400mVrms |
| Polarity | When a DC source’s “+” polarity is attached to speaker’s “+” polarity, and “-” polarity is attached to speaker’s “-” polarity, the membrane will move towards “-” orientation. |  |  |

## 1.3.4 Nominal Frequency Response Curve

Figure 1

Test signal: 20Hz～20kHz, 1/12 Oct, IEC standard 711 coupler

| Frequency (Hz) | Upper limits (dB) | Lower limite (dB) |
| --- | --- | --- |
| 20-4000 | +3 | -3 |

## 1.3.5 THD

Figure 2

Test signal: 20Hz～20kHz, 1/12 Oct, IEC standard 711 coupler

| Frequency (Hz) | THD limits (%) |
| --- | --- |
| 50-8000 | 2.5 |

## 1.3.6 TS parameters

| T/S | Numerical | Unit |
| --- | --- | --- |
| Mms | 0.023 | g |
| Fs | 270 | Hz |
| Sd | 0.52 | cm2 |
| Re | 16.43 | Ohm |
| Qm | 0.482 |  |
| Qe | 1.203 |  |
| Qt | 0.375 |  |
| Rm | 0.081 | kg/s |
| BL | 0.613 | N/A |
| Vas | 0.1049 | l |
| X-max | 0.55 | mm |

# Part list

| PART NO. | PART NAME | Q'TY | MATERIAL |
| --- | --- | --- | --- |
| 1 | DIAPHRAGM | 1 | PEEK+CU |
| 2 | FPC | 1 | CU |
| 3 | VOICE COIL | 1 | CU |
| 4 | FRAME | 1 | PPA |
| 5 | Magnet1 | 1 | Nd-Fe-B |
| 6 | PLATE | 1 | SPCC |
| 7 | Magnet2 | 1 | Nd-Fe-B |
| 8 | Yoke |  | SPCC |

# Reliability Test Criteria

## After any following tests the response at 1KHz shall not deviate more than ± 3dB from the initial value.

| ## No | ## Item | ## Test Conditions |
| --- | --- | --- |
| 1.5.1 | #### High Temperature Life | Temperature: 70℃ Duration: 96hrs, power off |
| 1.5.2 | #### Low Temperature Test Storage | Temperature: -25℃ Duration: 96hrs, power off |
| 1.5.3 | #### Humidity Test | Temperature: 40±3℃ Humidity: 95%RH Duration: 96hrs, power off |
| 1.5.4 | #### High/ Low Temperature load | #### 60°C, 8 hours, at rated power #### -20°C, 8 hours, at rated power |
| 1.5.5 | #### Drop Test | In X, Y and Z directions, the height of 1.20M falls 3 times in each direction (The products without front cover are placed in the protective shell) |
| 1.5.6 | #### Temperature Shock | Temperature: -25±3°C, 25±3°C, +70±3°C Time:10 cycles |
| 1.5.7 | #### Long Term Load Test | #### pink noise #### 5mW（0.28V）for 96 hours |
| 1.5.8 | #### Salt Spray Test | 5% concentration sodium chloride solution PH value 6.7-7.2 Solution temperature in the experiment box 35±2°C Air temperature of the experiment box is 47±2°C Time is 24 hours |
| 1.5.9 | #### Short Term Maximum Power Load Test | #### pink noise, Max power, last for 1 second power on and 59 seconds power off, 60 times |

# Package

## 200pcs speakers in each tray

## 5 trays in one inner pack

## 10 inner packs in one big carton