| Name | Security Level |
| --- | --- |
| BELLSINGTM Receiver In The Canal Assembly | Open |
| Version | A Total of 4 Pages |
| REV A |  |

BCS320H00002

Product Specification

| Prepared by: | Jack Zhao |  | Date: | 2022/9/15 |
| --- | --- | --- | --- | --- |
| Auditor: | Nick Zheng |  | Date: | 2022/9/15 |
| Approver: |  |  | Date: |  |

Shenzhen Bellsing Acoustic Tech. Co., Ltd.

| Product number :BCS320H00002 (2Pin Socket Assembly, Black) |  |
| --- | --- |

Product Specification

Contents

- Description and application……………………………………………………………..….……2

  - Description………………………………………………………………...………………..2

  - Application……………………………………………………………………………….....2

- Mechanical Layout Dimensions………………………………………………………………....3

  - Main dimension………………………………………………………………………..…....3

  - Material List…………………………………………………………………………..…….3

- Specification………………………………………………………………………………..4

- Change History……………………………………………………………………… ….………5

# Description and Application

1.1 Description: Connector socket assembly for Hearing aid BTE side

1.2 Application: Consumer electronics and medical equipment

# Mechanical Layout and Dimensions

## 2.1 Main Dimension

BCS320H00002

## 2.2 Material List

BCS320H00002

| Item No. | Description | Quantity | Material |
| --- | --- | --- | --- |
| 1 | Connect Pin | 2 | CuZn35Pb2 |
| 2 | Socket Housing | 1 | PA66, Black |

# Specification

| MECHANIAL DATA |  |
| --- | --- |
| INSERTION FORCE (SOCKET/PLUG): |  |
| SOCKET (TERMINALS) | >3.0 N (0.7 lb.) |
| EXTRACTION FORCE (SOCKET/PLUG): |  |
| SOCKET (TERMINALS) | <8.0 N (1.8 lb.) |
| ELECTRICAL DATA |  |
| CONTACT RESISTANCE | <150 mΩ |
| CONTACT RESISTANCE STABILITY (PULLING WITH 100g IN ALL DIRECTIONS) | <150 mΩ |
| INSULATION RESISTANCE | >1 mΩ |
| CURRENT (CONTINUOUS) | = 500 mA MAX. |

# Change History

| Revision | Specification Changes | Date | Responsible |
| --- | --- | --- | --- |
| A | Frist Release | 2022/9/15 | Jack zhao |