| Name | Security Level |
| --- | --- |
| BELLSINGTM Hornet Series AMBA Receiver | Open |
| Version | A Total of 4 Pages |
| REV B |  |

BRC360C10012

Product Data Sheet

| Prepared by: | Zoe Wang |  | Date: | 2018-2-28 |
| --- | --- | --- | --- | --- |
| Auditor: |  |  | Date: |  |
| Approver: |  |  | Date: |  |

Shenzhen Bellsing Acoustic Tech. Co., Ltd.

BRC360C10012 AMBA receiver

# Description

## Cylindrical Advanced Miniature balanced armature（AMBA） receiver.Smallest full-range driver, distinct bass and crystal clear treble.This transducer can also play an excellent role as a tweeter in commercial hearing products,communication earpieces.

| ## Name: | ## Hornet Series AMBA Receiver |
| --- | --- |
| ## Part Number: | ## BRC360C10012 REV B |

# Construction Specification

## Weight:  0.16 g

## Mechanical dimension：Please refer to the drawing below

# Features&Benefits

## Perfect package size for Headset/Bluetooth,TWS headset applications

## Outstanding efficiency

## Substantial maximum output

## Magnetic-radiation shielded

## Optimized mechanical shock resistance

# Electro-acoustic Specifications

## Test condition

## Acoustic tubing: 10.0mm of 1.0mm diameter tubing

## Acoutic coupler: IEC711 coupler

## Constant voltage drive: 0.084V RMS

## Environment: 23℃, RH 50%

## Acoustic Parameter I

| Test Item |  | Nominal | Tolerance | Comments |
| --- | --- | --- | --- | --- |
| Sensitivity(dB) | @200Hz | 107.5 | +/- 3.0 |  |
|  | @500Hz | 107 | +/- 3.0 |  |
|  | @1000Hz | 108 | +/- 3.0 |  |
| Peak1 | Frequency (Hz) | 2240 | +/- 350 |  |
|  | SPL(dB) | 118.5 | +/- 3.0 |  |
| Peak2 | Frequency (Hz) | 4370 | +/- 500 |  |
|  | SPL(dB) | 113 | +/- 3.0 |  |
| Maximum output @ Peak frequency (dB) |  | N/A | N/A | THD<10% |

## Acoustic Parameter II

| Test Item |  | Nominal | Comments |
| --- | --- | --- | --- |
| THD(%) | @1/2 1st peak | <5.0 |  |
|  | @1/3 1st peak | <5.0 |  |

## Electric Parameter

| Test Item |  | Nominal | Tolerance | Comments |
| --- | --- | --- | --- | --- |
| Impedance(Ω) | @500Hz | 14.5 | +/- 15% |  |
|  | @1000Hz | 20.5 | +/- 15% |  |
| DCR(Ω) |  | 12.0 | +/- 10% |  |

## Nominal Frequency Response Curve

# Package and Storage Condition

## Package: PET bubble pack, 1 pcs per bubble, 100 pcs per pack

## Storage temperature range: -40℃ ~ 60℃

## Storage humidity range: <60%