| Name | Security Level |
| --- | --- |
| BELLSING® BSP dynamic speaker | Open |
| Version | A Total of 5 Pages |
| REV A |  |

D16 dynamic speaker

BSP160CAB20

Product Data Sheet

| Prepared by: | Jerry |  | Date: | 2021-11-25 |
| --- | --- | --- | --- | --- |
| Auditor: | Allen |  | Date: | 2021-11-25 |
| Approver: | Allen |  | Date: | 2021-11-25 |

Shenzhen Bellsing Acoustic Tech. Co., Ltd.

Restricted

## 1.Security Warning

The information contained in this document is the exclusive proprietary to Bellsing Inc., and should not be disclosed to any third party without the written consent of Bellsing Inc.

## 2.Environmental Requirement

This s product including all components must be free from lead (Pb) and other banned substances according to customer's requirements.

## 3. Record Update

| Date | Version | Description | Editor |
| --- | --- | --- | --- |
| 11/25/2021 | A | Initial Release | Jerry |

BSP160CAB20 dynamic speaker

# Description

## This product works on the principle of permanent magnet. When electro-magnetic field and voice coil, on which a vibration membrane is attached, receive a current signal, the membrane vibrates and the air around the diaphragm will be compressed. Sound will be induced through such kind of compression and then the electronic sig- nal is transformed into sound signal.

## The BSP160 is a high performance Φ16.0mm dynamic speaker, recommaned to be used in consumer electronics audio products requiring high output in free air, such as smart glass.

# Mechanical Specification

## Mechanical dimension：please refer to the drawing below

# Electro-acoustic Specifications

## 1.3.1 Operation and storage environment conditions

## Operating Temperature Range: -20℃ to +60℃

## Storage Temperature Range: -20℃ to +65℃

## 1.3.2 Test Method

## 1.3.3 Electric-Acoustic Specification

| Test Item | Nominal | Tolerance | Comments |
| --- | --- | --- | --- |
| Impedance | 20 | ±15% |  |
| DC resistance | 19 | ±15% |  |
| Rated power | 20mW |  |  |
| Maximum shor-term power | 40mW |  |  |
| Resonance Frequency (F0) | 450±15%Hz in free air |  | 140mVrms |
| Sound pressure level | 118±3dB SPL at 1kHz / IEC711 |  | 140mVrms |
| SPL | See Figure1 |  | 140mVrms |
| THD | See Figure2 |  | 140mVrms |
| Frequency Range | 20~20kHz |  |  |
| Rub & Buzz | Must be free audible noise from 100 Hz to 2KHz |  | 630mVrms |
| Polarity | When a DC source’s “+” polarity is attached to speaker’s “+” polarity, and “-” polarity is attached to speaker’s “-” polarity, the membrane will move towards “-” orientation. |  |  |

## 1.3.4 Nominal Frequency Response Curve

Figure 1 Typical FR curve

Test signal: 20Hz～20kHz, 1/12 Oct, IEC standard 711 coupler

| Frequency (Hz) | 20-3550 | 3551-5000 |
| --- | --- | --- |
| Upper limits (dB) | +3 | +3 |
| Lower limits (dB) | -3 | -5 |

## 1.3.5 THD

Figure 2 Typical THD curve

Test signal: 20Hz～20kHz, 1/12 Oct, IEC standard 711 coupler

| Frequency (Hz) | THD limits (%) |
| --- | --- |
| 20~8000 | 3 |

## 1.3.6 TS parameters

| T/S | Numerical | Unit |
| --- | --- | --- |
| Mms | 0.023 | g |
| Fs | 450 | Hz |
| Sd | 1.24 | cm2 |
| Re | 18.73 | Ohm |
| Qm | 0.44 |  |
| Qe | 0.958 |  |
| Qt | 0.707 |  |
| Rm | 0.021 | kg/s |
| BL | 0.62 | N/A |
| Vas | 0.31 | l |
| X-max | 0.3 | mm |

# Part list

| PART NO. | PART NAME | Q'TY | MATERIAL |
| --- | --- | --- | --- |
| 1 | Diaphragm | 1 | PEEK+CU |
| 2 | Coil | 1 | CU |
| 3 | Frame | 1 | PPA+SUS301 |
| 4 | Plate | 1 | SPCC |
| 5 | Magnet | 1 | Nd-Fe-B |
| 6 | Yoke | 1 | SPCC |

# Reliability Test Criteria

## After any following tests the response at 1KHz shall not deviate more than ± 3dB from the initial value

| ## No | ## Item | ## Test Conditions |
| --- | --- | --- |
| 1.5.1 | #### High Temperature Life | Temperature: 70℃ Duration: 96hrs, power off |
| 1.5.2 | #### Low Temperature Test Storage | Temperature: -25℃ Duration: 96hrs, power off |
| 1.5.3 | #### Humidity Test | Temperature: 40±3℃ Humidity: 90-95%RH Duration: 96hrs, power off |
| 1.5.4 | #### High/ Low Temperature load | #### 60°C, 8 hours, at rated power #### -20°C, 8 hours, at rated power |
| 1.5.5 | #### Drop Test | In X, Y and Z directions, the height of 1.20M falls 3 times in each direction (The products without front cover are placed in the protective shell) |
| 1.5.6 | #### Temperature Shock | Temperature: -25±3°C, 25±3°C, +70±3°C Time:10 cycles |
| 1.5.7 | #### Long Term Load Test | #### pink noise #### 20mW（0.63V）for 96 hours |
| 1.5.8 | #### Salt Spray Test | 5% concentration sodium chloride solution PH value 6.7-7.2 Solution temperature in the experiment box 35±2°C Air temperature of the experiment box is 47±2°C Time is 24 hours |
| 1.5.9 | #### Short Term Maximum Power Load Test | #### pink noise, Max power, last for 1 second power on and 59 seconds power off, 60 times |

# Package

## 200pcs speakers in each tray

## 5 trays in one inner pack

## 10 inner packs in one big carton